James Webb Space Telescope

A STEM into Action resource funded by the UK Space Agency and managed by the D&T Association

# Context

These materials set out to improve pupils’ understanding of space and show how our existing knowledge has developed through a combination of observation, theory and technological advances. Pupils start by learning about the history of astronomy from the earliest optical telescopes to modern day space telescopes and use this knowledge to create their own telescope whilst learning about their design and operation. All of this takes place in the context of the James Webb Space Telescope which is due to launch in June 2019 and the likely uses for the data it gathers.

# Outcomes

After working through these materials pupils should

be aware of:

- the development of telescopes and how they are used to improve our understanding of the Universe.

understand:

- how telescopes work and how they are used.

- the design of telescopes including the James Webb Space Telescope.

- how the observations made using telescopes are used.

be able to:

- build their own telescope using low-cost materials.

- test telescopes and write reports making informed judgements about their performance.

- use their telescope to make observations on earth and in the night sky.

# Contents

James Webb Space Telescope	1

Context	1

Outcomes	2

Contents	2

Working in teams – (age 11-14)	4

Amateur astronomers	4

Who invented the telescope?	4

Refracting telescopes	5

How telescopes work	8

Calculating magnification	9

Cutting tubes	11

James Webb Space Telescope (JWST)	13

Locating celestial objects	15

Geometry – Cartesian coordinates	16

Geometry – Polar coordinates	16

Latitude and longitude	17

Mapping the universe – Ascension and Declination	17

Electromagnetic spectrum	18

James Webb Space Telescope instruments	19

JWST operation	21

Extended projects – age 14+	22

Refracting telescopes – Chromatic aberration	22

Light gathering	23

Diameter to area relationship	23

Reflecting telescopes	24

Mirror manufacture	24

Coating telescope mirrors	25

Eyepieces and pointing systems	26

Design Improvements	27

Remote operation!	30

GoTo telescopes – Computerised positioning	30

Digital imaging systems	31

Camera/phone adapters	31

The development of digital cameras	32

How digital cameras work	33

Digital camera sensors	33

Filters	34

Converting cameras to infrared	35

Camera modules for programmable systems	36

Telescope design	37

Image processing	38

Notes	40

Appendix one – links	41

# Working in teams – (age 11-14)

| Amateur astronomers |  | ENGINEERING |
| --- | --- | --- |
| Building and using your own astronomical telescope is a great way of gaining an awareness of the night sky and space observation. Amateur astronomers use a wide range of commercial and home-made equipment to view and capture images of objects in the Universe visible to people on earth. https://en.wikipedia.org/wiki/Amateur_astronomy |  |  |
| Even telescopes made from low-cost materials and plastic lenses will let you see objects on earth and in the night sky in greater detail. |  |  |

| Who invented the telescope? |  | HISTORY |
| --- | --- | --- |
| Scientists were using lenses to create telescopes as early as 1608 including Englishman Thomas Harriot who used a Dutch telescope to view the moon. Italian Galileo Galilei is recognised as the first person to use a telescope for astronomy in 1609. Galileo’s telescope had a magnification of 3x. He used a bi-convex lens for the objective and a bi-concave lens for the eyepiece causing viewed objects to appear the correct way up. https://en.wikipedia.org/wiki/Galileo_Galilei | https://www.universetoday.com/15763/galileos-telescope/ |  |

| Refracting telescopes Optics is the branch of physics which involves the behaviour and properties of light, including its interactions with matter and the construction of instruments that use or detect it. Wikipedia - https://en.wikipedia.org/wiki/Optics |  | SCIENCE |
| --- | --- | --- |
| Small, low-cost telescopes have two lenses, a larger ‘objective’ lens and smaller ‘eyepiece’ lens. The person viewing a distant object focuses the image by altering the distance between the two lenses. The telescope on the right has a rack and pinion gear moving the rear tube providing fine adjustment of the focus. | focus by turning knob |  |
| Parts of a telescope Card tube sliding inside larger tube Disc holding the small lens CENTRALLY IN the tube focus by sliding Eyepiece lens Objective lens Card outer tube |  |  |
| Prisms In science lessons you perform experiments to show how light rays passing through angled glass prisms change direction. Refraction Light travels through air and other transparent materials at different rates. This causes the light to be deflected or ‘refracted’ at the intersection between them. | triangular Glass prism Light Beam is refracted at the air/glass/air interfaces narrow beam of light | SCIENCE |
| Bi-convex lens parallel beams of light from a distant object The refractive index of the glass and radius of curvature determines how much the light will change direction. Light from distant objects enters the lens and, where this is at an angle to the glass, the light is refracted. The curved lens surfaces cause light rays from a distant object to converge at a single ‘focal point’. | focal length bi-convex lens Light beams converging focal + point |  |

| Low-cost lenses are made from optically clear thermoplastic polymers including polycarbonate and acrylic. Injection moulding allows high volume production which keeps the unit cost of each lens very low. Follow the link below to watch a video explaining how injection moulding machines work. https://www.youtube.com/watch?v=b1U9W4iNDiQ |  | ENGINEERING |
| --- | --- | --- |
| Most high-quality lenses are made from optical glass by several processes including cutting, grinding and polishing. Follow the link below to watch a video showing lenses being manufactured. https://www.youtube.com/watch?v=hX_YH0p_q9A |  |  |

| Sourcing lenses Lenses can be removed from old magnifying glasses and jewellers’ loupes. Make sure you get permission before dismantling magnifiers for their lenses! Opticians often sell magnifying glasses and jewellers’ loupes. |  |  |
| --- | --- | --- |
|  |  | DESIGN |
| Lenses can be purchased from different suppliers. Rapid Electronics, Breckland Scientific and Phillip Harris are three UK suppliers of STEM resources for schools and their catalogues list a range of bi-convex lenses. https://www.rapidonline.com/catalogue/search?Query=convex%20lens&Tier=Lenses http://www.brecklandscientific.co.uk/OLE-350-300-p/ole-350-300.htm https://www.philipharris.co.uk/search?phrase=lens |  |  |
| You will now need lightweight tubes with an internal diameter the same or slightly larger than your large, objective lens. The cardboard tubes some foods are supplied in may be suitable or those sold for sending posters through the post. Rolls of carpet often have a cardboard tube inside that might be suitable for your lenses. Builders merchants and DIY stores stock different sizes of plastic tube and these may be suitable for a low-cost telescope. Look in the sections for ventilation, plumbing and electrical. |  |  |

| How telescopes work |  |  |
| --- | --- | --- |
| The focal length of your lenses will determine how long your tubes need to be. If you purchased your lenses, you may already know the focal lengths. If not, it is easy to find out through a simple experiment. http://www.brecklandscientific.co.uk/OLE-350-300-p/ole-350-300.htm |  |  |
| Finding the focal length of a lens |  |  |
| Set up your lens like the photograph here facing light coming through a window. The lens can be clamped or held by BluTac. A vertical piece of paper on a wall or box will act as a screen for the image. The ruler zero mark should be aligned with the paper screen. Move the lens towards/away from the paper screen until the image of the window is in sharp focus. |  | SCIENCE |
| Make a note of the distance between the screen and the lens. This is the focal length for that lens. | focal length |  |
| Repeat the experiment for the eyepiece lens. The result of the experiments on the lenses in the photographs was: Focal length (objective) = 205 mm Focal length (eyepiece) = 75 mm |  |  |

| Calculating magnification Knowing the focal length of your lenses, it is possible to calculate the magnification for your telescope. Magnification = focal length of objective lens focal length of eyepiece lens |  | MATHEMATICS |
| --- | --- | --- |
| Example 1 Using focal lengths from the lenses above: Magnification = 205 = 2.7 75 Written as 2.7x magnification | Example 2 Using lenses with focal lengths of 1000mm (objective) and 40mm (eyepiece) Magnification = 1000 = 25 40 Written as 25x magnification |  |
| You should now be able to pair up lenses, choosing the focal lengths to give a specific magnification. Higher magnifications combine a long focal length objective lens and short focal length eyepiece lens. Magnification is only one consideration; equally important is the brightness of the image. This is determined by the diameter of the objective lens and how much light it can capture. The image seen through a telescope with a high magnification but small objective lens is likely to be dull and unclear. |  |  |

| Tube length |  | DESIGN |
| --- | --- | --- |
| The lenses need to be positioned in a telescope with their focal points in the same position. Allowing for a reasonable overlap between the tubes provides movement either side of this position to focus the image. https://lco.global/spacebook/refracting-telescopes/ |  |  |
| Example: With these lenses: Objective lens focal length = 300 mm Eyepiece lens focal length = 50 mm Lens distance = 300 + 50 = 350 mm | 350 mm Focus adjustment |  |
| If you want an overlap of 100 mm and both tubes are the same length, how long will the tubes need to be? | ??? mm ??? mm 100 mm overlap |  |
| Example: Half the overall length + half the overlap 175 mm + 50 mm Both tubes need to be 225 mm long. | half the overlap = 50 mm 175 mm mid point |  |
| You will need to repeat these experiments and calculations for your lenses. |  |  |

| You may be lucky finding tubes that slide smoothly inside each other. It is more likely you will have tubes with large gaps or only have one size of tube. In these cases it is relatively easy to choose one size of tube to fit the objective lens and reduce the diameter of the section that will slide inside. | DESIGN |
| --- | --- |

| Reducing tube diameter 84 Ø We could keep cutting narrow strips lengthwise out of a tube until we achieve the required diameter to achieve a snug fit. A knowledge of geometry lets us achieve an accurate outcome much quicker. Using measurements from the first tube: | 80 Ø 76 Ø 80 Ø TUBE CUT JOIN EDGES | MATHEMATICS |
| --- | --- | --- |
| Circumference=pi x diameter C=3.142 x 84 C=3.142 x 80 C=264 mm C=251 mm Difference=264-251=13mm A 13mm strip needs to be cut from the tube to reduce the outside diameter from 84mm to 80mm | 13 mm |  |
| You will need to repeat these experiments and calculations for your lenses. |  |  |

| CAD analysis Computer modelling offers an alternative to mathematical calculation. CAD software can be used to measure the dimensions of objects including circles and cylindrical shapes. Using Autodesk Inventor: - Click to activate the Inspect tab. In the Measure group, click Loop. Select the edge to be measured. The measurement will be displayed. |  | ENGINEERING |
| --- | --- | --- |

| Cutting tubes |  | DESIGN |
| --- | --- | --- |
| Before using any workshop tools make sure you have been shown how to use them safely. Carefully read the risk assessments and follow the control measures at all times. Gents saw with fine teeth | Craft knife Safety rule |  |
| - Mark out two lines showing where the segment needs to be cut out. A straight edge made from angled section makes drawing lines parallel to the axis of tubes easy. - Multiple cuts with firm pressure using a craft knife against a safety rule will cut through thin card tubes. |  |  |
| - Make sure the tube is held securely for cutting. You could rest the tube across the open jaws of a woodworking vice. |  |  |
| - Thicker card and plastic tubes are best cut using a fine tooth saw designed for cutting wood and soft plastics. A Gent’s back saw with very fine teeth is ideal for this. If you don’t have a Gent’s back saw, you could use a dovetail or Tenon saw. |  | DESIGN |
| When checking the tubes slide smoothly, use elastic bands to hold the edges together. Use an appropriate adhesive to glue the cut edges together. Use elastic bands to hold the edges together while the adhesive sets. |  |  |
| Cut a disc the correct size to hold the eyepiece lens in the end of the smaller diameter tube. Glue the lens into the hole in the disk. Make sure you keep glue away from the surfaces of the lens. Glue the disk into the end of the smaller tube. Depending on the materials, electrical tape may be appropriate for this joint. |  |  |
| Glue the objective lens into the end of the larger tube. Electrical tape may be suitable for this joint. |  | DESIGN |
| Slide the smaller tube inside the large one with the lenses at the outer ends. Check the tubes slide smoothly. |  |  |

| Testing - Using the knowledge you have gained, design tests to measure the performance of your telescope. Carry out the tests, recording the results. Create a report outlining the benefits and limitations of your telescope. | SCIENCE |
| --- | --- |

| Add a section to your report suggesting improvements to the design. Sketch the changes adding notes explaining: - the materials that could be used and the properties that make them suitable. - how the parts would be made. - How the modifications would work. Describe how these changes would improve the performance of your telescope. | DESIGN |
| --- | --- |

| James Webb Space Telescope (JWST) |  | ENGINEERING |
| --- | --- | --- |
| The James Webb Space Telescope is due to launch in June 2019 and has been designed to find evidence that will extend our understanding of the Universe is several areas. It will be used to study the first stars and galaxies by looking at the infrared spectrum. To achieve this the instruments must be cooled to within a few tens of degrees above Absolute Zero or - 273°C. This is to prevent radiation from the telescope and its instruments swamping the faint and distant astronomical signals. |  |  |
| UK involvement UK involvement in space includes: |  |  |
| The Science and Technology Facilities Council (STFC) is a UK government body that carries out civil research in science and engineering, and funds UK research in many areas including space science and astronomy. The STFC funds the UK Astronomy Technology Centre (UK ATC) with headquarters at the Royal Observatory in Edinburgh. They design and build instruments for many of the world's major telescopes including the Mid Infrared Instrument (MIRI) for the JWST. More than 20 partners are developing MIRI including the Rutherford Appleton Laboratory, Astrium Limited, the University of Leicester. | https://www2.le.ac.uk/departments/physics/research/src/res/mech_eng/present-missions/miri-project |  |
| University College London’s Mullard Space Science Laboratory helped with the development of the on-board and ground calibration equipment and another sensor on JWST, the Near Infrared Spectrometer (NIRSpec). UCL has partnered with Astrium on several contracts including the JWST and Gaia, ESA’s next generation astrometry mission to map the Galaxy, which launched in December 2013. http://www.ucl.ac.uk/impact/case-study-repository/supporting-the-growing-uk-space-sector |  | ENGINEERING |
| A Staffordshire-based company, Tekdata Interconnect Systems, is manufacturing a ‘cryogenic harness’ for the telescope. This crucial component will link all the JWST’s major systems. https://www.tekdata-interconnect.com/blog/drive_me_to_the_moon.asp |  |  |
| Follow the link below to learn more about the JWST through an interactive 3D model. https://jwst.nasa.gov/Webb3d/# Note: The interactive model requires Java so will not work with the Google Chrome or Microsoft Edge browsers. |  |  |
| A series of YouTube videos of the JWST: https://www.youtube.com/results?search_query=behind+the+webb%3A More information is available on these web sites: UK Space Agency NASA ESA |  |  |
| Careers A great many people are involved in designing, building and preparing the JWST across three partner organisations; NASA, ESA and the Canadian Space Agency. You will find more information on the JWST team here: https://jwst.nasa.gov/meettheteam.html | Professor Gillian Wright | CAREERS |

| ## Visiting speakers A network of STEM Ambassadors is available to work with UK schools on all aspects of science, technology, engineering and mathematics (STEM). https://www.stem.org.uk/stem-ambassadors/ambassadors |  | CAREERS |
| --- | --- | --- |
| ## Amateur astronomy A number of organisations support amateur astronomy in the UK including: The British Astronomical Association - https://www.britastro.org/ The Royal Astronomical Society - http://www.ras.org.uk/ The Society for Popular Astronomy : http://www.popastro.com/main_spa1/ |  | ASTRONOMY |

| Locating celestial objects When you look up at the night sky you are seeing the Universe from a rotating planet, the Earth. Looking up at the night sky on a clear night we see a constantly moving panorama of celestial objects. To keep a celestial object in view, the telescope must be constantly moving and an understanding of 3D geometry will help when locating objects and following their movements. | SCIENCE |
| --- | --- |

| Geometry – Cartesian coordinates |  | MATHS |
| --- | --- | --- |
| In mathematics you have learned about graphs, axes and coordinates. The graph here has x and y axes and any point on the graph can be described with x and y values. The numbers representing a point on the graph are called coordinates. For example, the point on the right has a 2D x,y coordinate of 8,4 |  |  |
| Add a third axis at right angles to the x and y and you have a 3D graph. This can also be used to locate points in 3D space. The point on the right has a 3D x,y,z coordinate of 5,4,3. y axis These are called Cartesian coordinates, named after the 17th century French philosopher, mathematician and scientist, Rene Descartes. ‘Cartesius’ is a translation of the Latin ‘Descartes’. | 4 3 5 z axis x axis | MATHS |

| Most 3D CAD software uses Cartesian coordinates to calculate the position of geometry in the virtual workspace. |  | ENGINEERING |
| --- | --- | --- |

| Geometry – Polar coordinates |  | MATHEMATICS |
| --- | --- | --- |
| An alternative method of plotting the position of objects on a 2D plane uses angles and distances. These are called polar coordinates. Two examples are shown on the right. 5,300 3,2250 | angle reference 3 5 30o 225o 3, 225o 5, 30o 0,0 (origin) |  |
| Early 2D CAD software defined the horizontal line to the right of the origin as the reference for angles measured anticlockwise. |  |  |

| # Latitude and longitude |  |  |
| --- | --- | --- |
| How do we know where we are on the surface of the earth? Early geographers devised a system using angles for latitude Ø (measured from the equator) and longitude λ (measured from the Greenwich meridian) to locate any position on the surface of the earth. | https://en.wikipedia.org/wiki/Latitude | GEOGRAPHY |

| Mapping the universe – Ascension and Declination |  |  |
| --- | --- | --- |
| Astronomers use a similar system to locate the position of celestial objects in the sky. Ascension is equivalent to longitude and is measured in hours (h) minutes (m) and seconds (s) with 24 hours equivalent to 360 degrees. Declination is the equivalent of latitude and uses the units of degrees °, arc minutes ', and arc seconds ". Declination north of the celestial equator uses +ve values with –ve values being used for points south of the celestial equator. https://en.wikipedia.org/wiki/Declination | Right ascension (blue) and declination (green) as seen from outside the celestial sphere. | SCIENCE |
| Sirius A is the brightest star in the sky and has coordinates: Right Ascension: 06h 45m 08.9s Declination: -16° 42’ 58” |  |  |

| Electromagnetic spectrum The electromagnetic spectrum describes the radiated energy that is all around us from very short wavelength Gamma radiation at one end to Long wavelength waves at the other. The diagram below shows the broad spread of the EM spectrum and expands the area around visible light. | SCIENCE |
| --- | --- |
| Visible and infrared wavelengths are the area of interest on this graph. The orange graph shows wavelengths the human eye is sensitive to. Notice how responsive it is at its peak. CMOS camera sensors, shown here with a green line, are about half the sensitivity of the human eye but can detect a much wider range of wavelengths. |  |

| To properly represent what humans see, digital cameras have an infrared blocking filter fitted immediately in front of the sensor. The blue line in the above graph shows how this type of filter allows only visible wavelengths through to the sensor. The image on the right was taken with an infra-red blocking filter. http://www.spitzer.caltech.edu/images/2154-sig08-004-Hands-in-a-Bag-color-Visible-vs-Infrared-Light |  | SCIENCE |
| --- | --- | --- |
| Infrared photography uses an infrared pass filter that cuts out visible wavelengths. The red line on the above graph shows the sensitivity of the S306 infrared pass filter. The image on the right was taken with an infra-red pass filter. http://www.spitzer.caltech.edu/images/2154-sig08-004-Hands-in-a-Bag-color-Visible-vs-Infrared-Light |  |  |

| James Webb Space Telescope instruments |  | SCIENCE |
| --- | --- | --- |
| The James Webb Space Telescope has four main instruments. These instruments are located in the Integrated Science Instrument Module (ISIM) and at the heart of each one are the sensors. The diagram on the right is from 3D computer model showing the location of the instruments. |  |  |
| Near-Infrared Camera (NIRCam) |  |  |
| The Near-Infrared Camera (NIRCam) is JWST’s primary imager and operates in the wavelength range from 0.6 to 5 μm. It consists of two, nearly identical, fully redundant modules. Each one has a pair of sensors detecting short and long wavelengths. https://jwst.stsci.edu/instrumentation/nircam |  |  |
| Near-Infrared Spectrograph (NIRSpec) |  |  |
| JWST's Near-Infrared Spectrograph (NIRSpec) is a near-infrared (wavelengths of 0.6 to 5.3 microns) spectrometer. Spectrographs tell us the intensity of the light coming from objects at different wavelengths. Studying spectra can tell astronomers many things about a celestial object, including what they are made of, how hot they are, and how fast they are moving. http://sci.esa.int/jwst/45694-nirspec-the-near-infrared-spectrograph/ |  |  |
| Mid-Infrared Instrument (MIRI) |  |  |
| The Mid-Infrared Instrument (MIRI) provides imaging and spectroscopic observing modes covering the wavelength range from 5 to 28.5 micron. MIRI’s scientific goals include, but are not limited to, direct imaging and spectroscopy of young exoplanets and their atmospheres, identifying and characterising the first galaxies in the Universe, and probing warm dust and molecular gas in young stars and protoplanetary disks. http://www.backstagescience.com/videos/MIRI.html |  |  |
| Near InfraRed Imager & Slitless Spectrograph (NIRISS) |  |  |
| This instrument is packaged with the fine-guidance camera and is capable of wide-field spectroscopy. NIRISS is poised to contribute to all of the JWST Science Themes: - Looking 13.5 billion years back in time. - The assembly of galaxies. - The birth of stars - Planetary systems and the origin of life. |  |  |

| JWST operation |  | ENGINEERING |
| --- | --- | --- |
| Light beams are directed to the different instruments in the JWST using mirrors. The light is reflected from the primary mirror to the secondary one, then through the optics assembly where fine steering mirrors reflect and split the light and direct it to the science instruments by pick-off mirrors. https://www.youtube.com/watch?v=y9Z2GbFJWmo |  |  |
| To give you an idea of how big the JWST is, watch a video of it being tested at NASA’s Goddard Space Flight Centre in Maryland, USA. https://www.youtube.com/watch?v=ASbkxmacK-4 |  |  |

| The James Webb Telescope is designed to record radiation in the range 500 – 5000nm. Draw a new line on the graph below to represent this. | SCIENCE |
| --- | --- |

# Extended projects – age 14+

This section provides pupils engaged in extended projects with suggestions for further investigation and study.

| Refracting telescopes – Chromatic aberration |  | SCIENCE |
| --- | --- | --- |
| If you have been following this guide from the beginning, you will have built your own telescope using two bi-convex lenses. You may have found it difficult to get a sharp image and noticed blurring and rainbow effects around the edge of objects. This is called chromatic aberration and happens when light passes from the air into the lens and out again. | https://en.wikipedia.org/wiki/Chromatic_aberration |  |
| In science you will have carried out an experiment using a beam of light and a triangular prism. This clearly shows the way white light is split or ‘dispersed’ into different colours. http://microscopy.berkeley.edu/courses/TLM/optics/chromatic.html |  |  |
| Think of it like a car driving from a tarmac road at an angle into sand. The tyre on the first side to cross onto the sand is slowed down and this turns the car slightly. Once all four wheels are on the sand, the car continues in a straight line. | the wheel on the right slows down when it enters the sand turning the car to the right tarmac Sand |  |
| The diagram on the right shows what is happening to the different colours of light passing through a lens. Different wavelengths (colours) focus at different places. Trying to focus your telescope on one colour leaves others blurred. https://en.wikipedia.org/wiki/Chromatic_aberration |  |  |
| One solution to the problem of chromatic aberration is to pair lenses with different refractive indexes. In this example, a bi convex lens made from crown glass is combined with a plano-concave lens made from flint glass. The different colours of light now focus very close to the same point. https://en.wikipedia.org/wiki/Chromatic_aberration |  | ENGINEERING |

| Light gathering One of the improvements you might consider for your telescope is increasing the amount of light entering your telescope. Using a larger objective lens will achieve this and the reasons for this are described below. | DESIGN |
| --- | --- |

| Diameter to area relationship A small increase in the diameter of a circle has a significant impact on its area. Doubling the diameter of the objective lens increases the area four times. This is because, in the area equation, the radius is squared. |  | MATHS |
| --- | --- | --- |
| Below are calculations swapping an 80mm diameter objective lens for one 160mm diameter. Area of a circle=π x r2 | 160 Ø 80 Ø |  |
| 80mm diameter lens Lens area=3.142 x 402 Lens area = 5027 mm2 | 160mm diameter lens Lens area=3.142 x 802 Lens area = 20,108 mm2 |  |

| Reflecting telescopes Below are listed the main advantages and disadvantages of each type of telescope. |  | DESIGN |
| --- | --- | --- |
| Advantages – Refracting - A sealed tube means dirt cannot easily get inside. - A sealed tube cuts down air movements inside and a sharper image results. - Lighter. - More robust. Disadvantages – Refracting - Viewing objects overhead places the eyepiece close to the ground. - Chromatic aberrations. - Higher cost. | Advantages – Reflecting - No chromatic aberration. - Easier to manufacture. - Lower cost for larger scopes. - Larger mirrors are possible. Disadvantages – Reflecting - Open construction allows dirt onto mirrors. - Air movements between the mirrors can cause distortion. - Less robust so more easily damaged. |  |
| Newtonian telescope The most popular design of reflecting telescope is called a Newtonian after Sir Isaac Newton who is credited with building the first telescope of this design in 1668. The light paths for this type of telescope are shown opposite. http://www.bbc.co.uk/schools/gcsebitesize/science/edexcel/visiblelight_solarsystem/telescopesrev3.shtml |  | SCIENCE |

| Mirror manufacture This section compares the manufacture of mirrors for optical telescopes with those in the JWST which are designed to capture near infrared wavelengths. |  | ENGINEERING |
| --- | --- | --- |
| The most critical part of a reflecting telescope is the primary mirror which is parabolic in shape. Optical telescope mirrors are manufactured by hand from low expansion glass by cutting, grinding, polishing and coating. If you would like to learn more, Gordon Waite has posted a series of videos on YouTube showing the entire process. https://www.youtube.com/user/GordonWaite/videos |  |  |
| The primary mirror in the JWST is made up of 18 hexagonal mirrors made from a very light but strong metal called Beryllium. Each hexagonal mirror for the JWST is machined from a solid piece of metal. The back of the mirrors is machined to form very thin sections and webs to create a rigid but lightweight structure. https://www.youtube.com/watch?v=ZDysmTVoC-k |  | ENGINEERING |
| Traditionally, optical telescopes on earth have a single, very large primary mirror. This video shows one being manufactured at a lab at the University of Arizona. https://www.youtube.com/watch?v=MjUcBWYVF9s |  |  |
| Polishing of the front surface of the JWST mirrors is done by computer-controlled machines and can be seen in the video linked below. https://www.youtube.com/watch?time_continue=4&v=Y_U_MrWcCnE |  | ENGINEERING |
| Coating telescope mirrors |  |  |
| Once a mirror is the correct shape, the surface needs to have a reflective coating applied. The links below explain the process for optical telescopes. http://www.orionoptics.co.uk/OPTICS/mirrorrecoating.html https://www.youtube.com/watch?v=MnkmQ_2ri9E |  |  |
| The mirrors on the JWST are coated in gold which is the most suitable material for reflecting infrared radiation. https://www.youtube.com/watch?v=fgn7bKs042Q |  | ENGINEERING |
| As technology advances, we may see other processes being used to coat mirrors. Company 3DDC has pioneered a process called Metalise which applies a thin coat of metal to 3D printed parts. In the future it may be possible to adapt this process for coating telescope mirrors. http://www.3ddc.eu/index.html |  |  |

| Eyepieces and pointing systems |  | SCIENCE |
| --- | --- | --- |
| Astronomical telescopes have interchangeable eyepieces with different magnifications. Eyepieces with a lower magnification and wider field of view are suitable for observing large objects like the Moon whilst higher magnification eyepieces will show more detail on small, distant objects like the rings around Saturn. |  |  |
| Especially at large magnifications, it can be tricky to find objects in the sky. A small ‘finder scope’ attached to the main telescope has a wider field of view making it easier to point the instruments in the right general direction to begin with. https://www.telescopeplanet.co.uk/national-geographic-50-600-refractor-telescope | finder scope |  |
| Some astronomers use laser pointers to help orientate their telescopes. |  | SCIENCE |
| Another technique used by astronomers is to gradually increase the eyepiece magnification, centring and focusing the target in the viewfinder before changing to a higher magnification eyepiece. https://www.tringastro.co.uk/eyepieces--barlows-15-c.asp |  |  |

| Design Improvements With a better understanding of how telescopes work you may want to improve yours. Here are some ways of improving the performance of telescopes. |  | DESIGN |
| --- | --- | --- |
| Tube connectors Connectors allow the use of different diameter tubes and can be designed for a smooth sliding motion. Connectors can also be designed to prevent the tubes coming apart when extended. |  |  |
| Focusing Sliding the tubes precisely to focus the image can be hit and miss. Using a rack and pinion gear will allow fine adjustment. Construction kits like Lego are a source of precision gears or you can use CAD software to generate gears which could be laser cut or 3D printed. |  |  |

| Stands/tripods |  | DESIGN |
| --- | --- | --- |
| A sturdy stand is essential when using telescopes especially at high magnification. Serious astronomers use steel and concrete to create permanent bases for their telescopes. Folding tripods and portable stands come in many different shapes and sizes and provide plenty of scope for exploring alternative design and methods of manufacture. http://www.rocketroberts.com/astro/observatory.htm |  |  |
| Pan/tilt movement This type of mount is ideal when observing things on earth but causes problems when moving the scope by hand when viewing stars. Panning is a term used in movies and videos and lets you view panoramas and follow things from side to side like birds in flight or vehicles on the move. To keep stars in view as the earth rotates you have to tilt and pan the telescope at the same time. | tilt pan http://www.rothervalleyoptics.co.uk/celestron-astromaster-az-tripod.html |  |
| Equatorial mount |  |  |
| Stars appear to move in the night sky due to the rotation of the earth. Equatorial mounts were designed with one axis (ascension) parallel to the Earth’s axis. The other axis is at right angles to the Earth’s axis and measures declination. With this type of mount you can follow a star by adjusting only the right ascension axis. https://grabcad.com/library/detailed-planet-earth-model-1 http://www.astronomysource.com/2011/12/17/german-equatorial-mount-part-2/ | declination axis perpendicular to the Earth’s axis direction of rotation earth’s axis right ascension axis aligned parallel to earth’s axis |  |
| Most equatorial mounts have the adjusting handles extended on flexible shafts. This lets you adjust the telescope without causing vibrations. http://www.rothervalleyoptics.co.uk/skywatcher-eq2-mount-tripod.html | flexible handle extension flexible handle extension |  |
| Motorised axes for smooth movement |  |  |
| If the right ascension axis is motorised at the correct speed, the telescope will keep a star static in the field of view allowing extended observation and time exposure photography. The rotational speed to achieve this is 1 revolution in 24 hours. Early telescopes used a clockwork motor to move the mount. Modern telescopes use electric motors. http://www.rothervalleyoptics.co.uk/celestron-astromaster-motor-drive-unit.html |  |  |
| Motorising both axes removes the physical connection between the observer and the telescope completely and any vibrations this might cause. With remote control of the motors, and a camera or phone capturing the image, you can even observe stars from the comfort of your living room! The setup on the right uses a Raspberry Pi educational computer to control the movement. | https://danlr46.wordpress.com/how-to-remotely-control-camera-filter-wheel-and-mount-with-raspberry-pi3/ |  |

| Remote operation! If you fancy serious star gazing using a professional telescope, there are web sites where you can purchase time. Using a web browser you control the telescope, then capture and download the images that you have taken! http://www.itelescope.net/ |  | SCIENCE |
| --- | --- | --- |
| National Schools Observatory The Liverpool Telescope (LT) was developed by Liverpool John Moores University (LJMU) and 10% of the observing time is allocated to schools in the UK and Ireland. The National Schools' Observatory (NSO) was created to act as a link between schools and the telescope which is located on a small volcanic island off the coast of Africa. https://www.schoolsobservatory.org/ |  |  |
| Faulkes Telescope Project This project provides free resources for science education and access to a global network of robotic telescopes. http://www.faulkes-telescope.com/ |  |  |

| GoTo telescopes – Computerised positioning |  | ENGINEERING |
| --- | --- | --- |
| Fully computerised telescopes are now available. They will ‘find’ a chosen star from a database of thousands of celestial objects. Once the domain of professionals and enthusiasts, GoTo telescopes can now be purchased for less than £300. https://www.telescopeplanet.co.uk/ |  |  |
| Digital imaging systems With a suitable adapter, almost any camera can be used to capture images from telescopes. Cameras used for astro-photography range from mobile phones, low cost web cameras, right up to professional single lens reflex (SLR) and mirrorless cameras. |  |  |

| Camera/phone adapters |  | DESIGN |
| --- | --- | --- |
| Cameras Adapters that hold the camera in the correct position let you concentrate on positioning the telescope and getting the image in focus. This adapter was custom designed to connect a specific phone and telescope. The shape was designed using parametric CAD software and 3D printed. |  |  |
| With access to 3D CAD software and CNC/3D printing equipment, custom camera/phone adapters can be designed and manufactured. This phone adapter can be downloaded for 3D printing from Thingiverse, one of several online libraries of 3D models. https://www.thingiverse.com/thing:353135 |  | DESIGN |
| Most of the mounts for mid/top range cameras are relatively simple, connecting the lens mount on the camera to the eyepiece tube on the telescope. https://www.thingiverse.com/thing:2332543 |  |  |
| The sensors fitted inside digital cameras are available separately and can be fitted to telescopes. This Raspberry Pi camera module adapter can be downloaded from Thingiverse. https://www.thingiverse.com/thing:214466 |  |  |

| The development of digital cameras |  | SCIENCE |
| --- | --- | --- |
| Frenchman Louis Daguerre is credited with creating the first lasting photograph in 1837. The image was recorded on a silver-plated copper sheet that had been sensitised by coating with gaseous fumes. A number of chemicals were used including halogen, iodide, bromine and chlorine. Paris boulevard – Louis Daguerre circa 1838/39 https://www.thoughtco.com/an-illustrated-history-of-photography-4122660 |  |  |

| The daguerreotype was superseded by paper and then celluloid with silver bromide coatings. Both of these can be rolled onto spools which take up very little room in the camera. In 1900 the Eastman Kodak company introduced the ‘Brownie’, the first mass produced camera. |  | ENGEINEERING |
| --- | --- | --- |
| The first digital camera In 1975 Steven Sasson, an engineer at Eastman Kodak company, designed and built the first practical digital camera using a charged coupled device (CCD) sensor. https://petapixel.com/2010/08/05/the-worlds-first-digital-camera-by-kodak-and-steve-sasson/ |  |  |
| How digital cameras work |  |  |
| The link below explains clearly how digital cameras work. https://www.makeuseof.com/tag/technology-explained-how-does-a-digital-camera-work/ https://en.wikipedia.org/wiki/Digital_camera#/media/File:Disassembled_digital_camera.jpg |  |  |

| Digital camera sensors |  | SCIENCE |
| --- | --- | --- |
| Digital cameras use two main types of sensor; charge coupled (CCD) and complementary metal oxide (CMOS) devices. The main advantage of CMOS sensors is the low power requirements. Camera sensors come in many different sizes. https://en.wikipedia.org/wiki/Image_sensor_format |  |  |
| One of the highest resolution cameras sensors is this Canon APS-H format, 250 megapixel monster! https://newatlas.com/canon-250-megapixel-sensor/39290/ |  |  |
| Filters |  |  |
| Earlier in this guide we explained how digital cameras have wide bandwidth sensors with Infra-red blocking filters to allow only visible wavelengths through to the sensor. The diagram here shows the IR-blocking filter location on top of the sensor inside the camera. http://dew.globalsystemsscience.org/tools/digital-cameras-overview/ Even with a filter in place, some infra-red wavelengths reach the sensor and we can see this by viewing a TV remote control through a digital camera. |  |  |
| Point the camera lens at the window at the front of a TV remote control. Look at the image on the back of the digital camera at the same time as someone presses buttons on the remote. You should see the infra-red LEDs on the front of the remote flashing. |  |  |

| Converting cameras to infrared This involves the following steps: - Opening the case to reveal the sensor. - Removing the infrared blocking filter. - Re-assembling the camera. - Attaching an infrared pass filter to the front of the lens. Note: Before trying this you MUST get the permission of the camera owner. |  | ENGINEERING |
| --- | --- | --- |
| Web camera This YouTube video was created by the European Space Agency (ESA): https://www.youtube.com/watch?v=zCJsrh51Ac4 It explains how to convert a low-cost web camera to record infra-red images. |  |  |
| Digital camera Before disassembling a working digital camera you MUST get permission from your parents and the owner of the camera. To find out how your camera comes apart, search on YouTube with your camera model and ‘IR conversion’. This set of three YouTube videos shows how to convert a Lumix digital camera including adding an infra-red pass filter. https://www.youtube.com/watch?v=frtQxryDTo8&t=167s |  |  |

| Camera modules for programmable systems With schools moving to coding lessons, small, single board computers are increasingly being used in classrooms. Most of these have add-on camera modules that can be converted for infra-red photography. |  | ENGINEERING |
| --- | --- | --- |
| IR camera module for Raspberry Pi The Raspberry Pi NoIR Camera Module v2 is a high quality 8MP add-on board for Raspberry Pi, featuring a fixed focus lens. The board itself is tiny, at around 25 x 23 x 9mm and weighs just over 3g, making it perfect for mobile or other applications where size and weight are important. The camera is perfect for doing IR photography and taking pictures in low light environments. |  |  |
| http://cpc.farnell.com/raspberry-pi/rpi-8mp-noir-camera-board/raspberry-pi-noir-camera-board/dp/SC14029?ICID=I-Rpi-fp2 |  |  |
| IR camera module for Arduino LinkSprite’s serial port camera module can capture high resolution pictures and transport them over TTL serial, making it ideal for embedded applications. And it can do it in the dark, thanks to infrared LEDs. Resolution - 160 x 120, 320 x 280 and 640 x 480 pixels. |  |  |
| https://www.open-electronics.org/an-arduino-based-jpeg-camera-with-ir-and-pir/ |  |  |

| Telescope design Telescope design has developed over many decades and produced a wide range of shapes, sizes and layouts. The lack of one ‘ideal’ design indicates there is still plenty of room for innovation. Recent developments in materials, CNC machining and 3D printing provide even more opportunities for improving the performance of optical telescopes. |  | ENGINEERING |
| --- | --- | --- |
| Telescope bodies Refracting telescope Most refracting telescopes have a solid tube body. In addition to supporting the lenses in the correct positions, it prevents stray light and dust from entering and reduces air currents. |  |  |
| When choosing material for the tube, look for light weight, low cost and a low thermal expansion. These composite tubes are made to order for telescopes from paper and epoxy resin. http://www.gerdneumann.net/english/instrument-building-parts-teile-fuer-den-fernrohrbau/fernrohrtuben-aus-hartpapier-phenolic-telescope-tubes-hard-paper.html |  |  |
| Significant distortion can occur if the telescope is at a different temperature to the surroundings so choosing materials with good thermal conductivity for the main structure will help the scope acclimatise more quickly. The surface inside tubes is usually painted matt black and many scopes have ridges, all designed to cut down internal light reflections. http://www.protostar.biz/blite.htm |  |  |
| Reflecting telescope Some reflecting telescopes use tubular bodies. They are simple to make and support but can cause ‘tube currents’ which distort the image. Tube currents are caused by the tube warming the cold night air. Large tubes can take a long time to reach the same temperature as the environment. Choice of materials plays a large part in the time taken for the temperature to stabilise. http://naturalimagesgallery.com/atm/lh_scope/raw_tube_on_mount.html |  |  |
| Open tube or ‘truss’ designs allow air to flow freely through the instrument shortening the time for the temperature of the telescope to stabilise relative to the environment. Frame structures also tend to be lighter and can be disassembled to make transporting the telescope to remote dark sky sites easier. http://www.rothervalleyoptics.co.uk/meade-lightbridge-10-dobsonian-telescope.html |  | ENGINEERING |
| Image processing |  |  |
| Previously in these materials we highlighted capturing the maximum amount of light as the single most important factor in producing bright images of objects in space. For live observation this is determined by the area of the objective lens or primary mirror. |  |  |
| The next breakthrough in astronomy used photographic film and motorised movement to create time exposures. Light from a distant object was captured over an extended period of time and recorded onto photographic film. The image here is the first solar eclipse photograph taken on July 28, 1851 by a daguerreotypist named Berkowski. | https://en.wikipedia.org/wiki/Astrophotography |  |
| Digital imaging provided the next significant advance where several images of the same object are ‘stacked’ to create an enhanced image. This has the advantage of cutting down the random coloured dots or ‘noise’ on the image. Key to this is a telescope that can keep an object in view for an extended period during which several images are recorded. After the viewing session the images are combined which increases the intensity of the final image. Professional astronomers first developed this technique which is now commonly used by amateurs. | http://www.instructables.com/id/Astrophotography-Star-Photo-Stacking/ |  |
| Imagine being able to see the result of stacking images in real time. This is what a start-up company called Unistellar is doing with their innovative eVscope. Their system is based on a small (11cm diameter mirror) reflecting telescope with motorised axes and a sensitive electronic sensor. A database containing the position and movement of stellar objects is used to control the motors allowing it to ‘follow’ an object in the sky. |  | ENGINEERING |
| Unistellar developed software that captures images and, in real time, combines them into a single image which is projected into the eyepiece. When a faint object is first acquired the observer will see very little, but as more and more images are combined the object will slowly become brighter, sharper and more colourful. The eVscope is due to go on sale in the summer of 2018. http://unistellaroptics.com/ https://www.kickstarter.com/projects/unistellar/evscope-100-times-more-powerful-than-a-classical-t?ref=blog |  |  |

# Notes

JWST data - https://jwst.nasa.gov/about.html

- Named after James Webb, a NASA administrator who managed the organisation during the earliest days of manned space flight leading to Neil Armstrong and Buzz Aldrin walking on the moon.

- Collaboration between the USA (NASA), Europe (ESA) and Canada (CSA).

- Infra-red telescope.

- 6.5 metre primary mirror made up of 18 segments that unfold after launch.

- Tennis court size, five-layer sunshield.

- Cameras and spectrometers used to capture very faint signals.

- Ariane 5 launch in spring 2019.

- Designed to work for 5-10 years.

- Orbits the sun 1.5m km from earth, a position where it remains in the same relative position to the Earth.

- Aims to study the history of the Universe including the first luminous glows after the Big Bang, to the formation of solar systems capable of supporting life on planets like Earth, to the evolution of our own Solar System.

# Appendix one – links

| Focus | Web link |
| --- | --- |
| James Webb Space Telescope | https://jwst.nasa.gov/about.html |
| JWST YouTube Channel | https://www.youtube.com/user/NASAWebbTelescope |
| UK Space Agency | https://www.gov.uk/government/organisations/uk-space-agency |
| European Space Agency (ESA) | http://www.esa.int/ESA |
| NASA | https://www.nasa.gov/ |
| ESA YouTube Channel | https://www.youtube.com/channel/UCIBaDdAbGlFDeS33shmlD0A |
| Rapid Electronics (Supplier of lenses) Breckland Scientific (Supplier of lenses) | https://www.rapidonline.com/catalogue/search?Query=convex%20lens&Tier=Lenses http://www.brecklandscientific.co.uk/OLE-350-300-p/ole-350-300.htm |
| Phillip Harris (Supplier of lenses) | https://www.philipharris.co.uk/search?phrase=lens |
| University of Leicester | https://www2.le.ac.uk/departments/physics/research/src/res/mech_eng/present-missions/miri-project |
| UCL Mullard Space Science Laboratory | http://www.ucl.ac.uk/impact/case-study-repository/supporting-the-growing-uk-space-sector |
| Tekdata Interconnect Systems | https://www.tekdata-interconnect.com/blog/drive_me_to_the_moon.asp |
| Interactive 3D model of the JWST | https://jwst.nasa.gov/Webb3d/# |
| Meet the JWST team | https://jwst.nasa.gov/meettheteam.html |
| STEM Ambassadors | https://www.stem.org.uk/stem-ambassadors/ambassadors |
| The British Astronomical Association | https://www.britastro.org/ |
| The Royal Astronomical Society | http://www.ras.org.uk/ |
| The Society for Popular Astronomy | http://www.popastro.com/main_spa1/ |
| National Schools' Observatory | https://www.schoolsobservatory.org/ |
| Faulkes Telescope Project | http://www.faulkes-telescope.com/ |
| Unistellar eVscope | http://unistellaroptics.com/ |

Tim Brotherhood – February 2018